3lectricEy3  //  Phase 2: Network Enumeration

Concept & Observation Reference  //  Training Use Only

THE CONCEPT — Know Your Network

Enumeration is the process of mapping everything on the network before touching anything. Rushing past this step is the most common student mistake — teams that enumerate thoroughly find both paths, identify all targets, and make better exploitation decisions.

The exercise LAN contains three devices: the router (gateway), the IP camera, and the smart plug. Students must identify all three and understand how they relate before proceeding to Phase 3.

IMMEDIATELY AFTER JOINING THE NETWORK

| 1 | Confirm your IP Check what address DHCP assigned — this tells you the subnet |
| --- | --- |
| 2 | Identify the gateway The router's LAN IP — typically the first or last host in range |
| 3 | Scan the subnet Discover all live hosts — note every IP that responds |
| 4 | Port scan each host Identify open services on camera and smart plug |
| 5 | Map the topology Sketch or note how devices relate before exploitation begins |

READING THE SUBNET

Your assigned IP reveals the subnet. Common exercise ranges:

| Your IP | Subnet | Host range |
| --- | --- | --- |
| 192.168.1.x | 192.168.1.0/24 | 192.168.1.1 – .254 |
| 192.168.0.x | 192.168.0.0/24 | 192.168.0.1 – .254 |
| 10.0.0.x | 10.0.0.0/24 | 10.0.0.1 – .254 |

IDENTIFYING THE CAMERA

Once you have a list of live hosts, the camera will typically reveal itself by:

Open port 554 — this is the standard RTSP port

Open port 80 or 8080 — camera web interface

HTTP banner or title containing the manufacturer name (Foscam, Hikvision)

ONVIF service on port 3702 — common on IP cameras

⚠  Document the camera IP and open ports before moving to Phase 3.

IDENTIFYING THE SMART PLUG

Smart plugs vary by model but commonly expose:

Open port 80 or 443 — web control interface

Proprietary API port (varies by manufacturer)

UPnP discovery on port 1900

Note the smart plug IP — triggering it ends the evolution. Don't touch it until you are ready to conclude.

COMMON PORTS — WHAT THEY TELL YOU

| Port | Service | Likely device |
| --- | --- | --- |
| 80 / 8080 | HTTP | Router, camera web UI, smart plug |
| 443 | HTTPS | Secure web interface |
| 554 | RTSP | IP camera video stream |
| 23 | Telnet | Old router/camera management |
| 22 | SSH | Router or Linux-based device |
| 1900 | UPnP | Smart plug, camera discovery |
| 3702 | ONVIF | IP camera (WS-Discovery) |
| 49152+ | UPnP control | Smart home devices |

TWO PATHS — DECISION POINT

At this point in the exercise two valid paths exist. Your enumeration output should inform which you pursue first.

| Path A — Router First | Path B — Direct to Camera |
| --- | --- |
| Exploit the router admin interface first. Gain elevated access to the LAN before touching the camera. | Some cameras expose services directly on the wireless segment or run their own AP. Check for this during enumeration. |
| More steps — higher learning value for lateral movement. | Faster — rewards thorough recon. Bonus points if both paths mapped. |

FIELD NOTES — ENUMERATION

My IP address:       ___________________________

Subnet mask:         ___________________________

Gateway (router):    ___________________________

Camera IP:           ___________________________

Camera open ports:   ___________________________

Smart plug IP:       ___________________________

Plug open ports:     ___________________________

Other hosts found:   ___________________________

Path chosen:  □ Router-first   □ Direct camera

Both paths found?  □ Yes   □ No

PHASE 2 COMPLETION CHECKLIST

□  IP address and subnet confirmed

□  All live hosts identified and documented

□  Camera IP and open ports recorded

□  Smart plug IP recorded — do not trigger yet

□  Attack path chosen and documented

□  Ready to proceed to Phase 3 — camera exploitation

3lectricEy3  //  UNCLASSIFIED TRAINING  //  Controlled Environment Only