3lectricEy3  //  Phase 3: Camera Exploitation

RTSP Stream Access  //  Training Use Only

THE CONCEPT — RTSP

What is RTSP? Real Time Streaming Protocol — the standard used by IP cameras to serve live video to viewers such as NVRs, mobile apps, and media players. Many older consumer cameras expose their RTSP stream with weak or no authentication. Anyone on the same network segment can pull the feed directly without needing camera login credentials. Standard RTSP port: 554. Some cameras use 8554 or custom ports. Verify against your Phase 2 port scan output.

FINDING THE STREAM URL

RTSP stream paths vary by manufacturer. Common patterns for exercise cameras:

| Manufacturer | Common RTSP path |
| --- | --- |
| Foscam | /videoMain or /11 |
| Hikvision | /Streaming/Channels/101 |
| Dahua | /cam/realmonitor?channel=1 |
| Generic | /stream1 or /live or /video |

⚠  If common paths fail — check the camera web interface on port 80, or search the model number for its RTSP path.

DEFAULT CREDENTIALS TO TRY

| Manufacturer | Username | Password |
| --- | --- | --- |
| Foscam | admin | (blank) |
| Hikvision | admin | 12345 |
| Dahua | admin | admin |
| Generic | admin | admin or 1234 |

Try unauthenticated access first — many cameras don't require credentials for RTSP.

PHASE 3 PROCESS

| 1 | Confirm camera IP Pull from your Phase 2 enumeration notes |
| --- | --- |
| 2 | Identify RTSP port Port 554 standard — verify from port scan |
| 3 | Construct stream URL rtsp://[camera_ip]:554/[path from table] |
| 4 | Open in media player VLC: Media > Open Network Stream > paste URL |
| 5 | Try with credentials if needed Format: rtsp://admin:12345@[ip]:554/[path] |
| 6 | Confirm live video You should see the camera feed on screen |
| 7 | Screenshot with timestamp Evidence of access — required for scoring |
| 8 | Record exact stream URL You will need this for Phase 4 if attempting Tier 1 |

TROUBLESHOOTING

| Issue | Try this |
| --- | --- |
| Connection refused | Wrong port — try 8554, 80, or check scan |
| Stream not found (404) | Wrong path — try alternate paths from table |
| Auth required | Try default creds from table above |
| Video won't play | Try VLC if ffplay fails — codec difference |
| Stream connects but black | Camera may be offline — check power |

ENDING THE EVOLUTION — SMART PLUG

For Tier 2 completion: once stream access is confirmed, trigger the smart plug to power off the camera. This freezes the live feed and ends the evolution.

Access the plug via its web interface or API on the IP identified in Phase 2. Toggle power off. Cadre confirm.

⚠  Do not trigger the plug until you are ready to end — or until feed loop is active if attempting Phase 4.

PHASE 3 COMPLETION CHECKLIST

□  Camera IP confirmed from Phase 2

□  RTSP port identified

□  Stream URL constructed and documented

□  Live video confirmed on screen

□  Screenshot with timestamp taken

□  Stream URL recorded for Phase 4 (if Tier 1)

□  Smart plug IP confirmed and ready

□  Evolution ended via smart plug  OR  proceeding to Phase 4

FIELD NOTES

Camera IP:           ___________________________

RTSP port:           ___________________________

Stream URL:          ___________________________

Credentials used:    ___________________________

Stream confirmed at: ___________________________ (time)

Smart plug triggered: __________________________ (time)

3lectricEy3  //  UNCLASSIFIED TRAINING  //  Controlled Environment Only