3lectricEy3  //  Kismet Quick Reference

Alfa AWUS036ACH + Yagi  //  Field DF Card  //  Training Use Only

PREREQUISITES — Run Once Per Machine

1. Install Kismet & dependencies

sudo apt update && sudo apt install kismet

2. Install Alfa rtl8812au driver

sudo apt install dkms

git clone https://github.com/aircrack-ng/rtl8812au

cd rtl8812au && sudo make dkms_install

3. Add user to kismet group

sudo usermod -aG kismet $USER

Log out and back in after this step

4. Verify Alfa card is visible

iw dev

Look for wlan0 or wlan1 in the output

MONITOR MODE — Run Before Each Evolution

Kill interfering processes

sudo airmon-ng check kill

Start monitor mode

sudo airmon-ng start wlan0

Confirm monitor mode is active

iwconfig wlan0mon

Must show:  Mode:Monitor

KISMET SITE CONFIG — Set Once

Create this file to avoid manual config on every launch:

sudo nano /etc/kismet/kismet_site.conf

Paste these lines:

source=wlan0mon

log_prefix=/home/student/captures/

log_types=kismetdb,pcapng

pcapng log is used for AAR packet review

LAUNCH KISMET

kismet -c wlan0mon

Web UI opens at:  http://localhost:2501

⚠  Set password on first run — remember it

WHAT TO FIND IN THE WEB UI

| Field | What it tells you |
| --- | --- |
| SSID | Network name — identify exercise AP |
| BSSID | Router MAC — use with airodump-ng |
| Signal (dBm) | Your directional compass |
| Channel | Lock airodump-ng to this channel |

DIRECTION FINDING PROCESS — Yagi

| 1 | Omni scan first — identify target SSID and baseline dBm |
| --- | --- |
| 2 | Connect Yagi to Alfa — hold level, rotate slowly 360° |
| 3 | Note compass bearing where dBm is strongest |
| 4 | Move 50–100m perpendicular — take a second bearing |
| 5 | Triangulate intersection of two bearings = hide location |
| 6 | Repeat as you close in — signal will saturate near hide |

SIGNAL STRENGTH REFERENCE

| dBm | Strength | What it means |
| --- | --- | --- |
| -30 to -50 | Excellent | Very close — narrow search |
| -51 to -65 | Good | On bearing, keep moving |
| -66 to -75 | Fair | General direction only |
| -76 to -90 | Weak | Far or obstructed |

⚠  dBm closer to 0 = stronger. -40 is strong. -85 is weak.

FIELD NOTES

AP SSID: ___________________________

BSSID:  ___________________________

Channel: __________________________

Hide bearing 1 (from ___): _________

Hide bearing 2 (from ___): _________

Key recovered at: _________ IVs

Path taken:  □ Router-first   □ Direct camera

3lectricEy3  //  UNCLASSIFIED TRAINING  //  Controlled Environment Only